Road water harvesting practices in Ethiopia: Techniques and Approaches

Kifle Woldearegay1, Frank van Steenbergen2, Kebede Manjur1, Marta Agujetas Perez2, Taye Alemayehu2

(1Mekelle University, Ethiopia; 2MetaMeta, The Netherlands)

Abstract

Road development is one of the major investments in many countries in Sub-Saharan Africa (SSA). Roads often change runoff patters, leading to concentrated flow of water. Road hydraulic structures (road side drainages, culverts and bridges) are often designed to discharge concentrated flow of water from road catchments to ensure the safety of the roads. If not managed, water from roads could lead to a number of problems: flooding, water logging, gully erosion, and siltation/sedimentation. In hilly terrains water from roads could also contribute to road damages due to landslides. If properly planned/implemented, the negative effects of water from roads could be changed into positive. In the year 2013, a research consortium (MetaMeta research as a lead and Mekelle University and IDS-UK as members) in collaboration with different development partners in Tigray (particularly the Bureau of Agriculture and Rural Development) have started implementing a systematic approach of harvesting water from roads for groundwater recharge; the project was financed by UPGro Catalyst grant (NERC, UK). As a result, different but simple and locally adaptive techniques and approaches have been implemented in almost all parts of the Tigray region. This Catalyst grant has helped the research consortium members and the development partners to understand the potentials of water harvesting for livelihood improvements and resilience. To upscale this innovation, over the last years, we have been promoting road water management in different regions of Ethiopia and beyond with research support from NWO/WOTRO (The Netherlands), and from Global Resilience Program (GRP). This paper presents: (a) the approaches followed to systematically introduce and upscale road water harvesting, (b) the techniques implemented to harvest water from road catchments, and (c) the hydrological effects/benefits of the interventions. Harvesting/managing water from road catchments is found to have several benefits: increase in shallow groundwater recharge, improvement in soil moisture, increase in availability of surface water in ponds/reservoirs, and reduction in gully development. With the shortage of water in many parts of Ethiopia and in other countries in SSA, road water harvesting can be an instrument not only to create resilience to rainfall variability but also to ensure the safety of our infrastructures as unmanaged water from roads could ultimately damage the road itself. There is, therefore, a need to promote road water harvesting/management options in natural resources management, infrastructural development and environmental planning.

Keywords: Water harvesting, Roads, Ethiopia.